LED Integrated Testing System test report

| test item: | LED spectrum analyse |  |  |
| --- | --- | --- | --- |
| equipment: | Led test system |  |  |
| 设备厂商获得证书： | 国家半导体工程研发及产业联盟证书，中国光学光电子行业协会光电器件分会证书，高新技术企业证书 |  |  |
|  | Mode:MB 010 NRA 20061010 6000K | Manufacturer: |  |
| Test Identifier | Temperature:25℃ | Humidity:60% |  |
|  | Conner: | Date:2017.10 |  |
|  | Organ: |  |  |

curve of spectrum power distributing

| Spectrum |  |  | Electrical |  |  |
| --- | --- | --- | --- | --- | --- |
| λ(Peak): | 451.8 | nm | I(test): | 94.000 mA |  |
| λ(Main): | 488.9 | nm | Vf: | 220.200 V |  |
| λ(Centroid): | 0.0 nm |  | φv: | 737.98 lm |  |
| λ(Center): | 0.0 nm |  | Efficiency: | 72.351 lm/w |  |
| BandWidth: | 25.8 | nm | P: | 10.2 W |  |
| ColorTemp: | 6272 | K | PFC: | 0.493 |  |
| CIE(x,y): | 0.3115, 0.3285 |  | Ra1:82.4 | Ra2:89.5 | Ra3:93.7 |
| CIE(u,v): | 0.1972, 0.3119 |  | Ra4:83.9 | Ra5:83.6 | Ra6:84.8 |
| [[ |  |  |  |  |  |
| Ra: | 84.3 |  | Ra7:86.9 | Ra8:69.5 | Ra9:12.7 |
| Purity: | 0.077 |  | Ra10:74.5 | Ra11:85.2 | Ra12:57.1 |
| Light Power: | 1204.1921 mw |  | Ra13:84.2 | Ra14:96.8 | Ra15:77.3 |

Remark:

page 1 of 1	2017-10-10	15:37:14